VFR Flight Plans

Flight Service Station FSS

1 800 Wx Brief	1 800 992 7433

Weather Briefing

TFRs

File A flight Plan

When we fly we should let someone know when to expect us back

Tell someone on the airport

Cell phone		(Story available)

File a flight Plan

Process to file a Flight Plan

Filing a flight plan is not required by regulations; however, it is a good operating practice, since the information contained in the flight plan can be used in search and rescue in the event of an emergency.

Flight plans can be filed in the air by radio, but it is best to file a flight plan either in person at the FSS or by phone just before departing.

After takeoff, contact the FSS by radio and give them the takeoff time so the flight plan can be activated.

When a VFR flight plan is filed, it will be held by the FSS until 1 hour after the proposed departure time and then canceled unless:

- the actual departure time is received;

- or a revised proposed departure time is received; or at the time of filing, the FSS is informed that the proposed departure time will be met,

- but actual time cannot be given because of inadequate communication.

- The FSS specialist who accepts the flight plan will not inform the pilot of this procedure, however.

The pilot must close a VFR flight plan at the completion of a flight.

- This can be done by contacting the nearest flight service station (FSS) or other FAA facility upon landing.

The Flight Plan Form

The figure above shows the flight plan form a pilot files with the Flight Service Station. When filing a flight plan by telephone or radio, give the information in the order of the numbered spaces. This enables the FSS specialist to copy the information more efficiently. Most of the spaces are either self-explanatory or nonapplicable to the VFR flight plan (such as item 13). However, some spaces may need explanation.

Aircraft Block

Item 3 asks for the airplane type and special equipment. An example would be C-150/X,  /X which means the airplane has no transponder. A listing of special equipment codes is listed in the Aeronautical Information Manual (AIM).

Time Block

Item 6 asks for the proposed departure time in Universal Coordinated Time (indicated by the “Z”).

Local Time (Daylight)							EDT	CDT	MDT	PDT

Convert to Zulu Time – Add to Local				+4		+5		+6		+7

Convert from Zulu Time - Subtract from Zulu 	-5		-6		-7		-8

Altitude Block

Item 7 asks for the cruising altitude. Normally, “VFR” can be entered in this block, since the pilot will choose a cruising altitude to conform to FAA regulations.

Sec. 91.159 - VFR cruising altitude or flight level.

Except while holding in a holding pattern of 2 minutes or less, or while turning, each person operating an aircraft under VFR in level cruising flight more than 3,000 feet above the surface shall maintain the appropriate altitude or flight level prescribed below, unless otherwise authorized by ATC:

(a) When operating below 18,000 feet MSL and --

(1) On a magnetic course of zero degrees through 179 degrees, any odd thousand foot MSL altitude +500 feet (such as 3,500, 5,500, or 7,500); or

(2) On a magnetic course of 180 degrees through 359 degrees, any even thousand foot MSL altitude +500 feet (such as 4,500, 6,500, or 8,500).

Route of Flight Block

Item 8 asks for the route of flight. If the flight is to be direct, enter the word “direct;” if not, enter the actual route to be followed such as via certain towns or navigation aids.

Item 10 asks for the estimated time en route. In the sample flight plan, 5 minutes was added to the total time to allow for the climb.

Fuel Block

Item 12 asks for the fuel on board in hours and minutes. This is determined by dividing the total usable fuel aboard in gallons by the estimated rate of fuel consumption in gallons.

Close your Flight Plan

Remember, there is every advantage in filing a flight plan; but do not forget to close the flight plan on arrival. Do this by telephone with the nearest FSS, if possible, to avoid radio congestion.